3GPP TSG-SA WG3 LI Meeting #9 	Tdoc S3LI02_042

Amsterdam, Netherlands. 29 – 31 January 2002

23 January 2002

# Consistency within spec regarding SMS based Addresses

## Spec:			33.108v0.3.1

## Release:		Rel-5

## Source: 		Telcordia Technologies

## Document for:		Discussion.

## Summary

TS 33.108 already includes material to support the capability of reporting the SMS Originating Address and SMS Terminating Address parameters (see Table 6-5, Section 6.5.1.1), which are listed separately from the SMS Center address.  However, these two parameters are still missing in other parts of the specification.  This contribution proposes additions to TS 33.108 to achieve consistency within the document.

## Proposal

- Add the SMS Originating Address and SMS Terminating Address parameters to Table 2.

| Parameter | Description | HI2 ASN.1 parameter |
| --- | --- | --- |
| SMS Originating Address | Identifies the originator of the SMS message. | DataNodeAddress |
| SMS Terminating Address | Identifies the intended recipient of the SMS message. | DataNodeAddress |

- Add parameters in the ASN.1 to indicate the parties to the SMS communication.

Proposal:  Add the following parameters under iRIParameter

sMSOriginatingAddress [34?] DataNodeAddress OPTIONAL,

sMSTerminatingAddress [35?] DataNodeAddress OPTIONAL,

- Add E.164 encoding to the DataNodeAddress in the ASN.1 to indicate the parties to the SMS communication can be E.164 addresses or IP addresses.

DataNodeAddress ::= CHOICE

{

ipAddress 	[1] IPAddress,

x25Address 	[2] X25Address,

e164-Format	[3] OCTET STRING    (SIZE (1 .. 25)),

-- E164 address of the node in international format. Coded in the same format as

-- the calling party number parameter of the ISUP (parameter part:[5])

...

}