3GPP TSG SA WG3LI	Tdoc S3LI01_135

October 30 - November 1, 2001

Aspen, Colorado, USA

# Source:	Siemens AG

# Title:		SRNS Relocation in case of PLMN change

# Document for:	Decision

Subject: 		Event for SRNS Relocation

# Introduction:

In case a subscriber roams into a PLMN and has to be intercepted this will be reported by sending necessary IRI records.

Following scenarios are possible:

a) Intercepted subscriber roams to a PLMN and switches on his mobile:

- Attach Event and RAU Event are generated

b) Intercepted subscriber roams to a PLMN and has his mobile already switched on:

- RAU Event is generated

c) Intercepted subscriber roams to a PLMN, has his mobile switched on and at least one 
    PDP context activated:

- RAU Event, Start of Interception with active PDP context Event

d) Same scenario like c) but with ongoing data transmission:

- RAU Event, Start of Interception with active PDP context Event are generated, but it takes some time to detect this event. Intercepted packets will be sent some time earlier on X0_3 interface than the events on X0_2. The reason to do this is that in this case not the "inter SGSN change" or "RAU" procedure is performed but the SRNS relocation procedure.
New to be documented in 33.107 is the fact that data on X0_2 will be sent some time later than data on X0_3.

Following message flow will illustrate this:

# Message flow for SRNS relocation

A: First Uplink user data can be received and have to be buffered.

B: First Downlink user data can be received and have to be buffered.

C: Correlation Number (GGSN address and charging ID) is now

available, delivery to LEMF on X3/X0_3 interface can start.

D: RAU event is detected, 'RAU' and 'Start of Interception with

active PDP context' will be sent on X2/X0_2 interface to LEMF.